VillageConnect

Integrated Local Communication System

Connecting Rural Communities Without Internet

Technical Report

Computer Networks Project

Submitted by

Owais AbuSalah

Abdulah Dwekat

Anas Habash

Supervised by

Dr. Jihad Hamamra

Academic Year 2025–2026

# Abstract

This report presents VillageConnect — an integrated local communication system designed to provide essential network services to rural communities that lack reliable internet access. The project simulates a complete network infrastructure for a village of approximately 1,000 residents, offering Voice over IP (VoIP) telephony, a local web portal in Arabic, and centralized file sharing — all without requiring an internet connection.

The network was designed using a three-tier hierarchical architecture and implemented in GNS3 with Cisco IOS devices. A Ubuntu Server running in VMware hosts the application services: Asterisk PBX for VoIP calls, Apache HTTP Server for the community web portal, and Samba for file sharing. OSPF dynamic routing connects four VLANs providing logical separation of administrative, public, voice, and server traffic.

The system was tested extensively and all core features function correctly, including inter-VLAN routing, voice calls with echo testing, web portal access from all network segments, and secure file sharing. This project demonstrates a complete, production-ready network design that can serve as a template for real rural communication deployments.

# Table of Contents

1. Introduction

2. Project Objectives

3. System Requirements

4. Network Architecture

5. IP Addressing & VLAN Design

6. OSPF Dynamic Routing

7. Server Services

7.1 Apache Web Portal

7.2 Samba File Sharing

7.3 Asterisk VoIP

8. Installation and Configuration

9. Testing and Results

10. Challenges and Solutions

11. Future Enhancements

12. Conclusion

13. References

# 1. Introduction

Access to communication technology is a fundamental enabler of modern society. Yet more than three billion people worldwide still live in areas without reliable internet connectivity. Rural villages in particular struggle with limited access to information, education, health services, and even basic voice communication. Traditional telephone infrastructure is expensive to deploy, and mobile carriers often skip low-population areas entirely.

VillageConnect addresses this gap by providing a self-contained local network that delivers three essential communication services to an entire village community: voice telephony, a web portal for announcements and directory information, and file sharing for educational resources. The system requires no external internet connection — everything operates on a private local network hosted on a single server.

This technical report documents the complete design, implementation, and testing of the VillageConnect network. It covers the hierarchical network architecture, VLAN segmentation strategy, OSPF dynamic routing, and the three application services. The project was implemented using industry-standard tools (GNS3, Cisco IOS, Ubuntu Server, Asterisk, Apache, Samba) and demonstrates how open-source technology can empower underserved communities.

# 2. Project Objectives

The project aims to achieve the following goals:

Design and simulate a complete enterprise-grade local network in GNS3

Implement proper VLAN segmentation for traffic isolation and security

Configure OSPF dynamic routing for automatic path calculation

Integrate a real Linux server (Ubuntu) into the simulated network

Deploy three production-ready services: VoIP, Web, and File Sharing

Provide an Arabic-language community portal with complete navigation

Enable voice calls between users using standard SIP protocol

Document all configurations, challenges, and solutions for reproducibility

# 3. System Requirements

## 3.1 Hardware Requirements

The host machine must meet the following minimum specifications to run the simulated network smoothly:

| Component | Minimum Specification |
| --- | --- |
| Processor | Intel i5 / Ryzen 5 or better with virtualization support |
| RAM | 8 GB minimum (16 GB recommended) |
| Storage | 40 GB free disk space |
| OS | Windows 10/11, macOS, or Linux |

## 3.2 Software Requirements

The following software is required to build and run the project:

GNS3 version 2.2 or later — network simulator

GNS3 VM — lightweight virtual machine for running Cisco images

VMware Workstation 17 Pro — virtualization for Ubuntu Server

Cisco IOS c3745 image — for the router

Cisco IOSvL2 image — for the switches

Ubuntu Server 24.04 LTS — for hosting application services

VPCS (built into GNS3) — for simulating end-user devices

# 4. Network Architecture

The network follows a classic three-tier hierarchical design, which is the industry standard for enterprise networks. This architecture separates the network into three layers, each with a specific role:

## 4.1 Core Layer

The Core Layer consists of a single router (Core-Router) running Cisco IOS on a c3745 platform. This router performs two critical functions:

Routes traffic between VLANs using sub-interfaces (Router-on-a-Stick technique)

Runs OSPF as the dynamic routing protocol to advertise all four subnets

The router connects to the Core Switch via a trunk link that carries traffic for all VLANs using IEEE 802.1Q tagging.

## 4.2 Distribution Layer

The Distribution Layer is implemented by the Core Switch (Core-Switch) running Cisco IOSvL2. This switch acts as the central hub connecting all access switches together and also provides connectivity to the Ubuntu Server (via a Cloud node that bridges to VMware).

Port Gi0/0: Trunk uplink to Core-Router carrying all VLANs

Ports Gi0/1-0/3: Trunk links to the three access switches

Port Gi1/1: Access port in VLAN 40 connecting to the Ubuntu Server

## 4.3 Access Layer

The Access Layer consists of three switches, each dedicated to one of the user VLANs:

Switch-Admin: Serves VLAN 10 (Administration)

Switch-Public: Serves VLAN 20 (Public Community)

Switch-Voice: Serves VLAN 30 (IP Phones)

Each access switch has one trunk uplink to the Core Switch and one or more access ports connecting end-user devices.

# 5. IP Addressing and VLAN Design

The network uses private IPv4 addressing from the 192.168.0.0/16 block. Each VLAN gets a dedicated /24 subnet, providing 254 usable host addresses per VLAN — sufficient for the village's population of approximately 1,000 residents distributed across the four functional groups.

## 5.1 VLAN Allocation Table

| VLAN ID | Name | Subnet | Gateway | Purpose |
| --- | --- | --- | --- | --- |
| 10 | Admin | 192.168.10.0/24 | 192.168.10.1 | Council & officials |
| 20 | Public | 192.168.20.0/24 | 192.168.20.1 | General residents |
| 30 | Voice | 192.168.30.0/24 | 192.168.30.1 | IP phones (QoS) |
| 40 | Servers | 192.168.40.0/24 | 192.168.40.1 | Backend services |

## 5.2 Host Assignments

The following specific IP addresses are assigned to key devices:

PC-Admin: 192.168.10.10 (example Administrative workstation)

PC-Public: 192.168.20.10 (example Public-network workstation)

PC-Voice: 192.168.30.10 (example Voice-network device)

Village-Server (Ubuntu): 192.168.40.10 (application server)

# 6. OSPF Dynamic Routing

OSPF (Open Shortest Path First) is a link-state routing protocol standardized in RFC 2328. It was chosen for this project because it is widely used in enterprise networks and provides several advantages over static routing.

## 6.1 Why OSPF?

Self-adapting: automatically recalculates routes when topology changes

Loop-free: uses Dijkstra's Shortest Path First algorithm

Fast convergence: typically under 30 seconds for small networks

Scalable: can handle hundreds of routers using areas

Industry standard: widely deployed in real-world networks

## 6.2 OSPF Configuration

The OSPF configuration on Core-Router advertises all four VLAN subnets into Area 0 (the backbone area):

router ospf 1

router-id 1.1.1.1

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.40.0 0.0.0.255 area 0

Since there is only one router in this simple topology, OSPF does not exchange routes with any neighbors — but the configuration is scalable for adding more routers in the future.

# 7. Server Services

All three application services run on a single Ubuntu Server 24.04 LTS virtual machine. This server is connected to the network through a VMware VMnet2 adapter which bridges to a Cloud node in GNS3, effectively placing the Ubuntu machine on VLAN 40 (the Server network).

## 7.1 Apache Web Portal

Apache HTTP Server hosts a community web portal written in HTML5 with Arabic (RTL) support. The portal consists of five pages accessible from any workstation in any VLAN:

| Page | Purpose |
| --- | --- |
| index.html | Homepage with village info and quick links |
| announcements.html | Official announcements from the village council |
| directory.html | Complete phone directory with all extensions |
| files.html | Instructions for accessing Samba shares |
| contact.html | Contact cards for key village services |

The portal uses responsive CSS with gradient backgrounds and can be accessed at http://192.168.40.10 from any device in the network.

## 7.2 Samba File Sharing

Samba provides SMB/CIFS file sharing, allowing villagers to access shared files from any Windows, macOS, or Linux computer using standard file browsers. Three shares are configured:

shared — read-only public library with books, videos, and school content

uploads — read-write folder for villagers to upload files

admin — password-protected folder for administrative files only

## 7.3 Asterisk VoIP

Asterisk PBX is the heart of the voice communication system. It runs on UDP port 5060 and manages five configured extensions:

| Extension | Name | Purpose |
| --- | --- | --- |
| 1000 | Admin | Village administration office |
| 1001 | Mayor | Village mayor's direct line |
| 1002 | Clinic | Health clinic |
| 2000 | School | School principal's office |
| 911 | Emergency | Emergency services line |

In addition, several service codes are provided:

*43 — Echo Test (plays back the caller's voice for audio testing)

100 — Announcement service (plays a pre-recorded message)

Users connect to Asterisk using any SIP softphone. For this project, Twinkle was used because it correctly handles SIP authentication and supports running multiple concurrent accounts for testing.

# 8. Installation and Configuration

## 8.1 Building the GNS3 Topology

The topology was built by dragging devices into the GNS3 workspace and connecting them according to the three-tier design. All Cisco devices received configuration via the console, and VPCS end-devices received their IP settings via simple ip commands.

## 8.2 Connecting Ubuntu to GNS3

Integrating the VMware virtual machine with the simulated GNS3 network required specific configuration:

Created VMnet2 as a Host-only network in VMware Virtual Network Editor (subnet 192.168.40.0/24, DHCP disabled)

Assigned Village-Server to use Custom: VMnet2 for its network adapter

Added a Cloud node in GNS3 renamed Village-Server

Configured the Cloud node to use VMware Network Adapter VMnet2

Connected the Cloud node to Core-Switch port Gi1/1 in access mode VLAN 40

## 8.3 Installing Server Services

With Ubuntu networking confirmed working, the three services were installed in sequence using standard apt commands. Configuration files were placed in /etc/asterisk/ for Asterisk, /etc/samba/smb.conf for Samba, and /var/www/html/ for Apache.

# 9. Testing and Results

Each component of the system was tested individually and then as an integrated whole. All tests passed successfully

.

| Test | Expected Result | Status |
| --- | --- | --- |
| Ping to gateway from all VLANs | Reply received | PASS |
| Inter-VLAN ping (Admin → Server) | OSPF routes correctly | PASS |
| Web portal from PC-Admin | HTTP 200 OK, pages render | PASS |
| Samba share listing | Files visible via smbclient | PASS |
| SIP registration (Twinkle) | Peer shows Unmonitored: 1 online | PASS |
| Echo test (dial *43) | Voice played back with echo | PASS |
| Call between extensions 1000 → 1001 | Ringing + answer + audio | PASS |

# 10. Challenges and Solutions

Several technical challenges were encountered and overcome during the implementation:

### 10.1 VMware LAN Segment Integration

Problem: LAN Segments defined in VMware do not appear in the GNS3 Cloud node interface selector, preventing the bridge between the virtual machine and the simulated network.

Solution: Replaced the LAN Segment with a VMnet2 Host-only adapter in VMware Virtual Network Editor. VMnet-style adapters are registered at the Windows network level and are visible to GNS3

.

### 10.2 Reverse Path Filtering

Problem: After establishing L2/L3 connectivity, ping requests from other VLANs reached Ubuntu but replies were dropped. Ubuntu's kernel considered the replies suspicious because the source IP (from another subnet) did not match the interface's subnet.

Solution: Disabled rp_filter permanently by adding net.ipv4.conf.all.rp_filter=0 to /etc/sysctl.conf and applying with sysctl -p. This is safe in a closed local network where source IP spoofing is not a realistic threat.

### 10.3 SIP Port Conflict

Problem: Twinkle softphone failed with "Failed to create a UDP socket on port 5060" because Asterisk was already bound to that port on the same Ubuntu machine.

Solution: Configured Twinkle's SIP profile to use alternative port 5062. SIP peers can use any port for registration, and Asterisk accepts connections to the standard 5060 regardless. of the client port

.

### 10.4 Linphone Authentication Failures

Problem: Linphone repeatedly received HTTP 401 Unauthorized responses despite correct credentials being entered. Packet capture showed Linphone was not sending the Authorization header on the retry.

Solution: Switched to Twinkle, which correctly implements the SIP digest authentication flow. Twinkle registered successfully on first attempt with the same credentials.

# 11. Future Enhancements

The current system provides a complete working foundation, but several enhancements could make it even more valuable for real-world deployment:

Wireless Access: add WiFi access points to allow smartphones and tablets to connect to the network

Video Conferencing: deploy Jitsi Meet for online classes and remote medical consultations

Offline Content: mirror Wikipedia, Khan Academy, and medical references for local educational access

Mesh Networking: use BATMAN-adv protocol to extend coverage across remote parts of the village

Solar Power: make network cabinets self-sufficient for areas with unstable electricity

Internet Gateway: add an optional satellite uplink for periodic synchronization with global services

Mobile App: Android app for the community portal (easier on phones than the browser)

# 12. Conclusion

VillageConnect demonstrates that a complete, production-quality local communication network can be built using open-source technology and standard networking hardware. The project successfully integrates a simulated network environment (GNS3 with Cisco IOS) with a real Linux server (Ubuntu in VMware) to deliver three essential services: voice calls, a community web portal, and file sharing.

The three-tier hierarchical architecture, VLAN segmentation, and OSPF dynamic routing reflect real-world enterprise network design practices. The Asterisk VoIP server, Apache web portal with Arabic RTL content, and Samba file shares show that a Linux server can run multiple production services on a single machine.

Most importantly, the project demonstrates how technology can serve underserved communities. A village with no internet can still have free local phone service, a central information portal, and a shared educational library — all using affordable, well-understood tools. This same design could be deployed in real villages around the world.

The team gained extensive hands-on experience in network design, Linux server administration, VoIP telephony, and troubleshooting complex integration issues. The challenges encountered — from VMware bridging to rp_filter to SIP authentication — mirror real problems faced by network engineers in production environments.

# 13. References

RFC 2328: OSPF Version 2 — J. Moy, 1998

Cisco IOS Configuration Guide, Cisco Systems, https://www.cisco.com

Asterisk Official Documentation, https://www.asterisk.org/documentation

Apache HTTP Server Documentation, https://httpd.apache.org/docs

Samba Official Documentation, https://www.samba.org

GNS3 Documentation, https://docs.gns3.com

Ubuntu Server Documentation, https://ubuntu.com/server/docs

Linux Kernel rp_filter documentation, Linux Kernel Documentation

Twinkle SIP Softphone, http://twinkle.dolezel.info

IEEE 802.1Q VLAN Tagging Standard